0:01

1 second

Today we are at the Portland Boat Show and in this video we're going to discuss everything that I wish I knew before I bought my first boat. Just kidding. That

0:09

9 seconds

was not my first boat, but it got your attention. We're going to be talking about heavy gauge aluminum boats in this video. Things to look for, features to look for, what to look for when buying a

0:17

17 seconds

used boat, and what you might want for your use case on a heavy gauge aluminum Northwest style boat. Now, most of these

Chapter 2: Size Classes

0:26

26 seconds

heavy gauge aluminum boats are going to be 18 foot and larger. There are a few 16 ft models out there. So, let's talk about a little bit about the differences

0:34

34 seconds

between the size classes of boats. The 16 to 18 footer class are great for lakes, bays,

0:43

43 seconds

decent days on most rivers, they're going to do just fine. Buoy 10, not sure I'd want to take anything smaller than

0:51

51 seconds

18 foot out to buoy 10. That's kind of dicey. I've seen it. But the weather can turn fast out there. Waves get big

0:58

58 seconds

really quick and the smaller sidewall height and the smaller transom depth yields to less safety margin when the

1:07

1 minute, 7 seconds

seas turn rough. Ocean, I wouldn't do that on an 18footer unless it was the calmst of calm days. Now, the 18 to 20

1:15

1 minute, 15 seconds

foot class of boat, we're starting to get into some real capability here. It can handle rougher weather. The sidewall

1:22

1 minute, 22 seconds

height is taller. Typically, it'll have a slightly sharper V, especially on the boats that have an offshore bracket.

1:28

1 minute, 28 seconds

Those will have a sharper V than the splash well smaller boats have, thus allowing for a smoother ride in rougher

1:36

1 minute, 36 seconds

water. And the taller transom height as well as taller sidewall height leaves you a lot more safety margin. And then there's the 21 plus class. Now, you have

1:43

1 minute, 43 seconds

to be careful with 21footers because some 21footers are just a long 20footer.

1:48

1 minute, 48 seconds

Some 21footers are just a short 22. In the case of the North Rivers, that's how it is. The and I think Thunderjet does this as well

1:56

1 minute, 56 seconds

where the 21 ft class, it gets anywhere between 3 and 6 in wider and your sidewalls get 3 to 5 in taller between

2:04

2 minutes, 4 seconds

the 20 foot and the 21 ft and up. It does come at a cost. These boats typically on the used market for 21 and

2:12

2 minutes, 12 seconds

up typically sell for at least $10,000 more than a 20-footer. But you're gaining extra sidewall height, extra

2:20

2 minutes, 20 seconds

transom height, sharper V. Well, in the case of North River, a heavier duty bottom plate on the hole, more fishing space. This is the class of boat you

2:29

2 minutes, 29 seconds

want at minimum, especially for buoy tan in the ocean. This class of boat is where you really start to get into some real capabilities of making sure that

2:37

2 minutes, 37 seconds

you can get home if weather turns south on you unexpectedly. Now, it's still not going to perform as well as the big

2:44

2 minutes, 44 seconds

offshore boats in really rough water, but it'll get you home. The biggest thing to know is that boats under 20 ft

Chapter 3: Foam in boats under 20ft

2:53

2 minutes, 53 seconds

must have reserved buoyancy foam in the boat somewhere enough to make the hole float. This is very important when we

3:01

3 minutes, 1 second

start talking about saltwater versus fresh water or the longevity of the hole. Some manufacturers have over the years put foam in their larger boats because it is an important safety thing.

3:13

3 minutes, 13 seconds

A lot of them have stopped in recent years though because it can promote corrosion in the boat. Now, the majority

3:20

3 minutes, 20 seconds

of Northwest style boats are going to be just like this one. Flat floor, open layout with a bench seat and a bench

3:29

3 minutes, 29 seconds

seat. And then up here we'll have either seat boxes like these two suspension seats or pedestal seats that are in

3:36

3 minutes, 36 seconds

fixed locations. That doesn't leave much room for foam. So they put it under the deck or they'll put it up underneath

3:44

3 minutes, 44 seconds

this bow compartment be uh in front of the console if they uh need more foam than what they can fit underneath the deck. Why is foam bad? Typically the

3:52

3 minutes, 52 seconds

foam is sprayed in. It's a two-part expanding foam that gets sprayed in at the factory and it it shapes itself to the hole and holds itself against the

4:01

4 minutes, 1 second

hole. What happens is when water eventually gets down in between the foam and the hole, it can't dry out. When it

4:08

4 minutes, 8 seconds

gets held there and it can't dry out, it starts to corrode the aluminum. This is made even worse in a salty environment

4:16

4 minutes, 16 seconds

like a lot of these boats get used in bays, buoy 10, the ocean, all of those

4:23

4 minutes, 23 seconds

are saltwater environment and especially in scenarios like crabbing where you are dragging salt water into the boat in

4:30

4 minutes, 30 seconds

reasonably large volumes for as far as water coming into the boat goes that doesn't involve your boat getting swamped. the water getting trapped in

4:38

4 minutes, 38 seconds

the foam with salt in it really becomes an issue and can start to corrode your boat away. So, while most manufacturers

4:46

4 minutes, 46 seconds

of 22 foot boats or 21 ft boats and larger have stopped putting foam in their holes, just make sure that if

4:54

4 minutes, 54 seconds

you're buying a used boat that's older, keep that in mind that it's something to check for and if it has foam, make sure it's not saturated and just plan on

5:02

5 minutes, 2 seconds

removing it if it's over 20 ft. Now, if your budget storage situation, towing situation, whatever has you stuck under

5:11

5 minutes, 11 seconds

20 feet, you're gonna have foam in the boat. And a lot of guys do remove it. I don't necessarily recommend that. I don't know the legality of it. Do that

5:19

5 minutes, 19 seconds

at your own risk, but that definitely will prolong the life of your boat, being able to actually wash all the salt

5:27

5 minutes, 27 seconds

out and get it off of the hole. So, let's talk about the whole shape and construction a little bit. uh on this Alum Weld Striker here. This is a

Chapter 4: Hull Shape and Construction

5:36

5 minutes, 36 seconds

smaller boat. I believe this is a going to be a 17 or an 18 foot. And these boats typically have,

5:57

5 minutes, 57 seconds

which is fine for this smaller class of boat. It's not going to be quite as robust as the larger boats, but it's going to survive some lighter hits.

6:08

6 minutes, 8 seconds

These boats are engineered to be tough.

6:10

6 minutes, 10 seconds

These smaller boats have a shallower V than the larger boats. Once you get up into the boats that have offshore brackets on them, either at 18t or 20

6:19

6 minutes, 19 seconds

foot and 21T plus boats, it's going to have a sharper V every time you go up in class.

6:27

6 minutes, 27 seconds

So if we look at this 22ft North River here with the offshore bracket, you can see that V at the transom is much

6:35

6 minutes, 35 seconds

sharper and the I think it's an 18 degree transom and the bow is 41°. It's a very sharp bow for cutting through the

6:44

6 minutes, 44 seconds

water because this 22ft North River is so much wider than the 17 or 18 foot aluma weld we were just looking at. If

6:51

6 minutes, 51 seconds

it had the same Vshape, it would pound a lot harder as it was going through the water. So, the steeper V helps it cut

7:01

7 minutes, 1 second

versus pound. Also, it being a wider hole means that it will be more stable even with the sharper V. So, you kind of

7:10

7 minutes, 10 seconds

get the best of both worlds as you go up in size of boat. Now, here we have a boat that's probably 20 to 22 feet long.

7:21

7 minutes, 21 seconds

It's going to be my guess. It might be a 23. This is an Alum Weld SuperV right here. And you can see the V on it is a

7:29

7 minutes, 29 seconds

lot shallower. So, this is going to have a harsher ride on plane on rougher water, but it's going to be extremely

7:37

7 minutes, 37 seconds

stable, and it's also going to run shallower. You'll also notice this boat has a flat spot known as a delta pad

7:46

7 minutes, 46 seconds

right here at the uh center of the transom.

7:50

7 minutes, 50 seconds

And what that does is it allows you to run a jet on your outboard for running in really really shallow water. Another

7:58

7 minutes, 58 seconds

important factor with the uh bottoms of the holes is not just the V, but the whole thickness. So you'll see on this

8:05

8 minutes, 5 seconds

22 FFT North River, this has a/4in thick plate of aluminum. So, if you hit something with this, it's a lot less

8:12

8 minutes, 12 seconds

likely to be damaged than the thinner plate that was on that smaller aluma weld that we were looking at earlier.

8:19

8 minutes, 19 seconds

Not all large boats use that thick of hole material. If we look at this 20ft duckworth advantage, this is going to

8:26

8 minutes, 26 seconds

have a 3/16inch thick plate on the bottom of the hole. Some manufacturers still use 316

8:35

8 minutes, 35 seconds

on their 21 plus class of boats. not necessarily the end of the world. They there's been a lot of boats out there

8:42

8 minutes, 42 seconds

with the 316 hole for a very long time that are still in great condition, but

8:49

8 minutes, 49 seconds

having the thicker hole material on the bottom is beneficial. Now, those thicker hole materials do come at a cost. One,

8:57

8 minutes, 57 seconds

monetarily, they're more expensive to manufacture and more expensive to purchase the materials for. It's also heavier, so that's more weight to tow

9:06

9 minutes, 6 seconds

and a larger tow vehicle that you may have to have. That's an important consideration depending on how you are going to be using your boat. Now, let's

Chapter 5: Offshore Bracket Styles

9:14

9 minutes, 14 seconds

talk offshore brackets versus splash walls for a minute. For 20 foot and under, a lot of times you'll find boats

9:21

9 minutes, 21 seconds

that have either a splash well or an offshore bracket. This one in particular, this Duckworth, has a splash wall. What that means is that when water

9:29

9 minutes, 29 seconds

hits the back of this boat and splashes over the top of the transom, say it's a wave or somebody's wake, it's going to

9:35

9 minutes, 35 seconds

come into this bowl right here and eventually wash down and out this hole

9:43

9 minutes, 43 seconds

that is in the transom. This keeps your boat from getting swamped. Now, over here on this North River, we have one

9:50

9 minutes, 50 seconds

style of offshore bracket. This one is a buoyancy bracket. So that means that the planing surface of the boat is extended

9:58

9 minutes, 58 seconds

out to where the motor is. And this bracket is actually part of what's floating the boat.

10:05

10 minutes, 5 seconds

This is a big deal. Uh what this does, this adds about 600 lb of flotation,

10:12

10 minutes, 12 seconds

which happens to be about the same weight as an outboard that's picking up on the outboard. Now, you'll notice

10:21

10 minutes, 21 seconds

the back of the boat right here. This is the back of the the main floating part of the boat. The outboard is right here.

10:30

10 minutes, 30 seconds

When we look at this boat, here's the back of the main floating portion of the boat. Look how much further back the

10:37

10 minutes, 37 seconds

outboard is. What that's doing is that's turning this point right here without that flotation. That's turning this point right here into a teeter totter.

10:49

10 minutes, 49 seconds

So that even at rest, the motor is trying to torque the boat this way. And

10:55

10 minutes, 55 seconds

uh that can contribute to pposing or when the bow of the boat does this as you're underway. Also without the

11:04

11 minutes, 4 seconds

flotation bracket, it will make the boat sit bow high at rest, especially in this

11:12

11 minutes, 12 seconds

class or this class of boats. And this thunderjet here has an offshore bracket

11:20

11 minutes, 20 seconds

that is a non-flotation bracket. So you can see the planing surface stops at the transom of the boat. So you have in this

11:28

11 minutes, 28 seconds

case two outboards that are acting as a lever on the fulcrum point here trying

11:35

11 minutes, 35 seconds

to lift the bow out of the water all the time, especially when you're underway.

11:41

11 minutes, 41 seconds

Now, less of an issue on this particular boat because you have the weight of all of this huge hard top cabin up here, the

11:49

11 minutes, 49 seconds

pilot house, but soft top boat that doesn't weigh very much, having a flotation bracket, is very important. If you're ever out trolling and you see a

11:58

11 minutes, 58 seconds

boat that doesn't have a flotation offshore bracket on it, especially one that's a soft top, look at it and you'll notice that it's at rest, it is sitting bow high instead of level in the water.

Chapter 6: Splashwell vs Offshore Brackets

12:11

12 minutes, 11 seconds

Biggest advantage to the offshore bracket versus a splash well is you have so much more space between the engine

12:20

12 minutes, 20 seconds

mount and the height of the transom of the boat.

12:25

12 minutes, 25 seconds

So, it has the water has to go so much higher and especially on this uh on the 21 plus class of boats, it's an extra 3

12:33

12 minutes, 33 seconds

in taller typically. So, it's even that much higher that water has to go before it can get into the boat. You have to be in a really bad situation to get water

12:42

12 minutes, 42 seconds

in the boat with an offshore bracket. On a splash wall boat, it's a little bit higher chance of getting water inside of the boat, especially on a splash well

12:50

12 minutes, 50 seconds

designed like this one. What we have here is a 20-in tall transom.

12:56

12 minutes, 56 seconds

The wing here is going to be 25 in, but still water can easily splash up and over. It could splash onto this and wash

13:05

13 minutes, 5 seconds

in or it could swamp the entire splash well and get into the boat because the front of the splash wall is only a

13:13

13 minutes, 13 seconds

couple inches taller than the transom itself. So, if a big rogue wave or a big wake hits the stern of that boat, higher

13:21

13 minutes, 21 seconds

chance of it getting swamped versus a boat that has an offshore bracket. Now, let's talk about kicker location. On these larger boats, there's a handful of

Chapter 7: Kicker Location

13:29

13 minutes, 29 seconds

ways to operate your kicker. Number one, I'm actually going to start over here on the Thunderjet. The kicker bracket is

13:36

13 minutes, 36 seconds

mounted to the offshore bracket and is steered at the steering wheel from the helm. This setup works great in the

13:45

13 minutes, 45 seconds

ocean and big open water. Does not work very well when you're trolling in heavy traffic like we do on the Columbia where having a tiller is beneficial, but is

13:54

13 minutes, 54 seconds

the most common in these big offshore boats cuz they're always out on the open water. Here we have an example of the

14:00

14 minutes

kicker being mounted on a bracket that's welded to the transom and it's much closer to the boat. Now, this setup is

14:09

14 minutes, 9 seconds

my favorite because the kicker is closer to the boat than it is on my boat. My boat is like that thunderjet over there

14:17

14 minutes, 17 seconds

with the kicker mounted on the end of the offshore bracket. This brings the kicker in closer for more comfortable time running the tiller and you can run

14:25

14 minutes, 25 seconds

a shorter extension in order to be able to reach it from the floor inside. You can also run these with a tie bar

14:33

14 minutes, 33 seconds

between the main motor and the kicker. I personally don't like to do that. I'm gonna say this is my second favorite setup that we're going to look at. Now,

14:40

14 minutes, 40 seconds

on this Duckworth that has the splash well, the kicker motor is mounted directly to the transom. And you can see

14:48

14 minutes, 48 seconds

the handle comes up right here. You've got easy access to reach it from inside of the boat. This is probably the most

14:56

14 minutes, 56 seconds

common setup for boats, well, this is definitely the most common setup for boats that don't have an offshore bracket. But there's something you're gonna notice about all three of these

15:04

15 minutes, 4 seconds

boats, and that's which side the kicker

15:11

15 minutes, 11 seconds

is mounted on. Doesn't really matter for this one. A lot of times manufacturers will mount the kicker on the port side

15:18

15 minutes, 18 seconds

or the left side of the boat because that's opposite of the driver to kind of offset the driver's weight. And I actually really disagree with that.

15:27

15 minutes, 27 seconds

Number one, in the case of my boat, when I'm on plane, the torque of the prop naturally rotates the boat to that side.

15:35

15 minutes, 35 seconds

So then I have to use my trim tabs to flatten the boat out. The other big issue is the location of the tiller

15:42

15 minutes, 42 seconds

handle on most kickers. Most kickers have the tiller handle on the left side of the motor here. Tiller handle on the

15:49

15 minutes, 49 seconds

left side of the motor. Tiller handle on the left side of the motor. I'm using this one specifically as an example. I

15:56

15 minutes, 56 seconds

used it in my small boat video as well because this shows the problem with having the kicker mounted on the left side and the handle on the left side.

16:05

16 minutes, 5 seconds

What happens when you have to turn left? You start hitting things.

16:13

16 minutes, 13 seconds

You have to pick the handle up in order to do a full lock turn, which in this case kind of hits the other motor and that's not a good thing. You have to

16:20

16 minutes, 20 seconds

kick the handle up in order to turn all the way if you have to make a really sharp turn. Additionally, if I'm standing here running the kicker, this is the only spot that I can stand.

16:31

16 minutes, 31 seconds

Period. I have to stand on the left side in order to have my hand on the tiller handle to operate the kicker. If the

16:39

16 minutes, 39 seconds

kicker was mounted over here, I could stand in the middle or and operate it with my right hand or I could stand over

16:47

16 minutes, 47 seconds

here and operate with my left hand depending on what I need to do given weight and balance and conditions. So

16:54

16 minutes, 54 seconds

here with the kicker mounted on the starboard side, you can see I can stand

17:00

17 minutes

over here. I can sit down on the gunnel, still reach the handle comfortably.

17:08

17 minutes, 8 seconds

I can stand in the middle right here facing this direction if I need to and operate the kicker. Also, when I turn

17:15

17 minutes, 15 seconds

full lock right, I'm not interfering with anything over here. Even if this had higher sides,

17:23

17 minutes, 23 seconds

same thing turning full lock left. I'm not interfering with anything and I'm not having to lean way over out the side of the boat. Especially in the case of a

17:32

17 minutes, 32 seconds

boat like mine that has to run an extension handle. Uh on my old boat, it was especially a problem. I had to lean way out over the side in order to turn

17:41

17 minutes, 41 seconds

the boat full starboard. I really think the kicker should be mounted on the starboard side. I don't know why every

17:48

17 minutes, 48 seconds

single manufacturer does it on the port side except for Alum Weld. Alum is the only ones that I know that actually put the kicker over here. Actually, I guess

17:56

17 minutes, 56 seconds

I shouldn't say that because a lot of manufacturers, at least of these sleds, have it so you can mount the kicker on

18:03

18 minutes, 3 seconds

either side, but I would still only put the kicker on the starboard side. So, I guess while I'm over here, we'll talk about tiller versus center console versus windshield boats.

Chapter 8: Tiller vs Center Console vs Windshield

18:14

18 minutes, 14 seconds

[snorts]

18:14

18 minutes, 14 seconds

Tiller boats like this one have the advantage of one operator station.

18:20

18 minutes, 20 seconds

The driver stands here at the main. Then he comes over here and operates the kicker. Doesn't have to go anywhere. One

18:27

18 minutes, 27 seconds

maybe two fish finders and they both mount into well in this case there's one fish

18:35

18 minutes, 35 seconds

finder in your electrical panel and then mount your second fish finder on the outside if you need a second one for uh life scope or something like that. but you have to buy less fish finder.

18:44

18 minutes, 44 seconds

Another advantage to an open sled like this is there is so much room for activities. A lot of times these sleds

18:52

18 minutes, 52 seconds

will be used for guides. Sometimes uh your regular everyday recreational fisherman will use these sleds as well though just because there's they're so versatile, so much room.

19:04

19 minutes, 4 seconds

They do river fishing really, really, really well. And on a calm day, they'll also do ocean fishing fairly well as well, except for when the wind starts to

19:12

19 minutes, 12 seconds

pick up and then spray will come on to your passengers and likely you also obviously you're exposed to the elements

19:20

19 minutes, 20 seconds

if you don't have your uh your biminy top up. Typically, these sleds will also have a fair amount of storage up here,

19:27

19 minutes, 27 seconds

as well as a lot of them will come with some semblance of seat boxes, which most guys like to run. I know some guys like to run the pedestal seats like this one

19:36

19 minutes, 36 seconds

because uh there's just a little bit smaller footprint. Some of these sleds will also come with uh rod storage

19:43

19 minutes, 43 seconds

underneath these uh in the gunnel doors here.

19:48

19 minutes, 48 seconds

Store fishing rods and that's a feature that you won't see on most windshield boats. Some

19:57

19 minutes, 57 seconds

windshield boats are starting to come with that but most of them it hasn't made it that way yet. center console boat. Obviously, we still have similar experience with single operator station.

20:10

20 minutes, 10 seconds

We have the advantage of having a steering wheel instead of the tiller handle to operate. Downside is you have this box in the middle of the boat.

20:21

20 minutes, 21 seconds

You can still get away with a single fish finder or two fish finders by just having it mounted up here. And it's easy to see back from the trolling station.

20:31

20 minutes, 31 seconds

Also, your net storage is very, very conveniently centrally located, which is nice. You get that's what this tube right here is. That's for his nets.

20:38

20 minutes, 38 seconds

Overall, it does give you a better view, I think, while you're driving and definitely more control having a steering wheel versus a tiller.

20:49

20 minutes, 49 seconds

Although, a lot of the larger tiller outboards now are coming with power steering. So, you rotate the handle a little bit and a hydraulic pump will

20:57

20 minutes, 57 seconds

actually do the turning of the motor for you. So, it's very similar to having hydraulic steering in that you don't have the torque of the motor trying to

21:06

21 minutes, 6 seconds

turn the the outboard on you. Same thing in this boat. We have seat boxes that are on adjustable sliders that can move

21:13

21 minutes, 13 seconds

back and forth. And this boat is set up for four clients. And then your captain and a deck hand can sit over here or a

21:20

21 minutes, 20 seconds

deckand can sit up there. And just like the aluma weld next door, this boat has some under bow storage. Now, let's talk

21:27

21 minutes, 27 seconds

about windshield boats. Big advantage, windshield keeps the wind off of you and rain when you're underway. Big

21:35

21 minutes, 35 seconds

disadvantage windshield. It makes it harder to see when you're operating a kicker.

21:42

21 minutes, 42 seconds

Also, it gets in the way if you want to fish off the bow for like bass fishing.

21:46

21 minutes, 46 seconds

You do have to contend with this for your back swing for your bass rod. If you're tall like me, these folding tops are a pretty big disadvantage on any

21:54

21 minutes, 54 seconds

boat. Uh on the sleds, they're actually even lower. Typically, they're about this tall, so you have to really crouch underneath them. It's meant to keep

22:01

22 minutes, 1 second

people dry while they're sitting and for the captain to look over the top of and see where he's going. So, it's a little bit less of a problem on a windshield

22:10

22 minutes, 10 seconds

boat because these are taller, but I still have to duck quite a ways in order to get inside of the top. Now, a big disadvantage of running a top is the

22:19

22 minutes, 19 seconds

visibility. When you're standing back here running the kicker, this is your view.

22:29

22 minutes, 29 seconds

I can't see much in front of me without actually ducking down unless I roll the top up, which is

22:37

22 minutes, 37 seconds

doable. But even still, you can unzip the top right here, unsnap the front, roll this up and keep the top up. But

22:46

22 minutes, 46 seconds

even still, it's reduced visibility here. Although, I will say it's a little it's slightly better on this boat than

22:53

22 minutes, 53 seconds

it is on my boat. So, here's here's another folding top on the next class up of boat. This is that 20 foot

23:02

23 minutes, 2 seconds

duckworth that we were looking at earlier. And you see I still kind of have to duck, although not quite as much as I did on the uh 17T aluma weld. And

23:11

23 minutes, 11 seconds

here you can get an idea of the visibility with the top kind of rolled up. And this boat actually,

23:19

23 minutes, 19 seconds

you see on the aluma weld over there, the the side windows go up much higher than this one. So, this one's actually blocking my visibility straight out uh

23:28

23 minutes, 28 seconds

to the sides without ducking down. And on my boat, this is even worse because

23:35

23 minutes, 35 seconds

it's like right here that the window starts cuz there's a piece of fabric that is between the top of the zipper

23:42

23 minutes, 42 seconds

and the window. So, my sidewalls actually come down to like right here.

23:47

23 minutes, 47 seconds

Visibility with my top up is absolutely terrible, which is why during fall salmon season, I actually take my top off so that I can see where I'm going.

23:58

23 minutes, 58 seconds

And then of course there's the pilot house. These boats are much larger. I'm not going to spend too much time on them, but I figured I'd mention it.

24:07

24 minutes, 7 seconds

A lot of times they Oh, is this locked? This is locked. Crap.

24:12

24 minutes, 12 seconds

Well, we'll just look in through the window here. Again,

24:20

24 minutes, 20 seconds

these typically get driven entirely up at the helm. Visibility out the back sucks. Sometimes, like this one in

24:26

24 minutes, 26 seconds

particular, has controls, at least controls for the kicker, steering for all of the engines, controls for the kicker at the rear station, and he's

24:35

24 minutes, 35 seconds

going to be looking through a window back here all the way through to the front to be able to see forward and poke his head around the side. that

24:44

24 minutes, 44 seconds

these boats are typically only used on the ocean or big open water, not so much in the Columbia River or they won't be

24:52

24 minutes, 52 seconds

used so much for trolling tight areas like the Ghost Hole and Telmuk Bay. These guys are going out to the ocean.

24:58

24 minutes, 58 seconds

All right, I had to come out to the shiny new boats to show you this. But here we have an intermediate hard top.

25:04

25 minutes, 4 seconds

So this is the same class of boat as my North River, but instead of being a soft top, this is a hard top.

25:14

25 minutes, 14 seconds

biggest difference you'll see between this and say an offshore boat is the windshield is leaning back towards the

25:22

25 minutes, 22 seconds

driver instead of forward. You you have the advantages of a hard top in that this is never going to leak, but you

25:30

25 minutes, 30 seconds

have the disadvantages of a windshield boat. So, also you can never take this top off. You're stuck with it. So, this

25:37

25 minutes, 37 seconds

what you have back here is your fishing area. I really like the versatility of my boat being able to take the soft top

25:45

25 minutes, 45 seconds

off and have more fishing area. It almost not quite but almost more like a sled than uh than these boats are

Chapter 9: What to look for on a used boat

25:54

25 minutes, 54 seconds

typically used. And finally, I want to talk about things to look for when you're shopping for a used boat. All of

26:02

26 minutes, 2 seconds

these things are going to kind of give you the totality of how the previous owner cared for the boat. You want to look for damage, especially on the

26:10

26 minutes, 10 seconds

bottom. see if it's had any hard impacts. That's not necessarily neglect, but if you see hard impact damage on the

26:18

26 minutes, 18 seconds

bottom, you want to make sure to look under the floor and make sure that there's no broken welds or structural damage underneath. It may not be an issue on boats that have a quarterinch

26:26

26 minutes, 26 seconds

bottom. But on boats that have a 3/16 bottom or even an eighth inch bottom, especially an eighth inch bottom, you're going to want to make sure that there's

26:35

26 minutes, 35 seconds

no structural damage if there is visible damage to the outside of the boat. Now you're gonna see dock rash. This boat I

26:42

26 minutes, 42 seconds

don't know if you can see. See if we can make that show up there. Yeah, this boat's got a dent in it. That's not necessarily pretty, but that's also not

26:51

26 minutes, 51 seconds

the end of the world. Every boat's going to have dock rash on it the first time it touches a dock. The age of the electronics, that's a big deal. None of

26:58

26 minutes, 58 seconds

the boats that I have been looking at out here for this video have old electronics. But if the electronics are old and outdated, you've got to take

27:06

27 minutes, 6 seconds

that into account that you might have to put a few thousand dollar into buying some new electronics. And if you have one or two fish finders at the helm and

27:14

27 minutes, 14 seconds

a third fish finder back at the kicker station, that price starts to add up really quick, especially when you start

27:21

27 minutes, 21 seconds

talking about things like radar, fleer, uh GPS navigation, autopilots, things like that. Like all of that really adds

27:30

27 minutes, 30 seconds

up when you have to replace it on somebody else's old boat. The condition of the interior will tell you a whole lot about how the previous owner took

27:37

27 minutes, 37 seconds

care of his boat. This 22 FFT North River in particular is very clean.

27:43

27 minutes, 43 seconds

There's there's a little bit of wear on the floor I can see, but there's I see one little tiny Oh, nope. That's that's just a leaf. I thought that was a spot

27:51

27 minutes, 51 seconds

that was gouged on the floor. This is very clean in here. I don't like the fact that it has carpet in the gunnel

27:59

27 minutes, 59 seconds

trays. Carpet holds salt water and that's not good. I also don't like the fact it has carpet on the outside of the boxes. That's just I don't like carpet

28:08

28 minutes, 8 seconds

in boats. It causes problems, especially in the salt water. But overall, this boat is extremely clean.

28:16

28 minutes, 16 seconds

There's uh there hasn't been a lot of leaking through of salt water through the zippers here down onto the control panel like on my boat. My control panel

28:24

28 minutes, 24 seconds

has some corrosion on it and this boat's not old enough. This is newer than 2017 or sorry, newer than 2016 because it has

28:32

28 minutes, 32 seconds

this style transom area. So, it's not that old and I don't see

28:38

28 minutes, 38 seconds

a tiny bit of corrosion right there some paint flaking off here, but overall

28:47

28 minutes, 47 seconds

Oh, here we go. Here's a little bit of corrosion right here.

28:51

28 minutes, 51 seconds

Surface corrosion up at the top. not that big of a deal. And you're going to have corrosion anywhere that you have stainless steel or water getting

28:59

28 minutes, 59 seconds

trapped. So in this case around these fittings around here, these rivets tend to get corroded out and then underneath

29:06

29 minutes, 6 seconds

the snaps. Sometimes if the snaps don't have this myar washer in between them and the aluminum, a lot of times you'll

29:15

29 minutes, 15 seconds

see corrosion in those points on certain manufacturers. I'm not going to call them out on video here, but certain manufacturers don't use those myar

29:22

29 minutes, 22 seconds

washers and uh it causes corrosion to start bubbling the paint. Other common points of corrosion on a lot of boats is

29:31

29 minutes, 31 seconds

right here, especially if this seam isn't welded at the edges of the paints.

29:35

29 minutes, 35 seconds

North Rivers after 2016, they started welding the chine down the outside on here. On my 2014, this isn't welded and the paint was all the way down there.

29:44

29 minutes, 44 seconds

That was a point of corrosion. So, uh, North River actually took my boat back to the factory, stripped of the paint,

29:52

29 minutes, 52 seconds

and put a wrap on it. Wraps are absolutely the way to go. Especially in salt water, paint, something hits it, it cracks, it gets a scratch in it, and

30:01

30 minutes, 1 second

water will start working its way underneath and sitting there, and you'll start getting corrosion bubbles. I'm going to look for some on this. Well,

30:09

30 minutes, 9 seconds

you can see uh here where the paint got actually scratched and eventually with

30:16

30 minutes, 16 seconds

use in salt water that can lead to corrosion bubbles. Also, you'll find corrosion in places like this that are

30:24

30 minutes, 24 seconds

just siliconed instead of welded. Salt water can work its way in there, sit and start corroding. Again, it's not the end

30:32

30 minutes, 32 seconds

of the world. It can be indicative of how the boat was used and how it was cared for. It's something to look for.

30:38

30 minutes, 38 seconds

If you see corrosion on the bottom plate of the boat or anywhere below the water line, if you see a lot of corrosion, right? Like this seam is not welded

30:46

30 minutes, 46 seconds

right here. If you see a lot of corrosion in these areas, that is indicative of future problems

30:55

30 minutes, 55 seconds

that are very expensive to repair. pay attention to to the corrosion, but don't necessarily be scared off, especially

31:02

31 minutes, 2 seconds

for anything that's above the water line or that's not attached to anything structural. Another common point of corrosion is on carpeted bunks,

31:10

31 minutes, 10 seconds

especially with saltwater use. So, if we look underneath here, yes, this bunk is only carpet.

31:19

31 minutes, 19 seconds

What happens is the salt water gets held in the carpet and uh you can't rinse it out without actually like dunking your

31:26

31 minutes, 26 seconds

trailer in fresh water. So if it takes you a little while to do that and you can't just rinse it off with a hose cuz

31:33

31 minutes, 33 seconds

it's up against the hole. So that salt water is sitting there and you'll have corrosion exactly where the bunks sit.

31:40

31 minutes, 40 seconds

The better solution is what's on this North River and I saw it on the Thunderjet there over there too is uh

31:47

31 minutes, 47 seconds

this has these rubber pads that sit on top of the carpet. And you'll notice there's light. I can get my finger in between the boat and the carpet here.

31:58

31 minutes, 58 seconds

So that allows and the those rubber pads are ribbed that lets you rinse the salt water out from there. So that way salt water is not sitting against the hole. A

32:06

32 minutes, 6 seconds

lot of guys want their boats to be shiny. Not sure why, but they don't like it when the aluminum has this dull look.

32:13

32 minutes, 13 seconds

And uh this is actually the natural state of the aluminum protecting itself.

32:18

32 minutes, 18 seconds

Exposed aluminum is safe aluminum. When the aluminum is painted, that's when water can get trapped. Now, wraps like

32:27

32 minutes, 27 seconds

this has if the wrap gets damaged, that's not the end of the world

32:35

32 minutes, 35 seconds

because then the aluminum is just exposed. You can see right here, the aluminum is exposed.

32:42

32 minutes, 42 seconds

But, uh, the where the wrap is not damaged, right next to it, it's seal.

32:48

32 minutes, 48 seconds

it's not going to get salt water stuck in it. Versus paint where it starts to bubble and then more and more water gets

32:55

32 minutes, 55 seconds

in there and starts the corrosion process and you get more and more stuck underneath the paint until you flake the paint off and then expose the aluminum.

33:02

33 minutes, 2 seconds

With a wrap, take the old wrap off, put the new wrap on and now it's all sealed again and it's just fine or it's

33:10

33 minutes, 10 seconds

exposed. There's no in between where it's not sealed but it's also not exposed trapping water. A couple of other points to look at when you're

33:19

33 minutes, 19 seconds

looking at a used boat. Is the build clean? Are the electronics clean? Is the wiring job good? You do not want to have a ground going through the hole. So,

33:28

33 minutes, 28 seconds

don't don't run a hot wire and then run your ground wire to the hole and then have the hole hooked up to the battery

33:34

33 minutes, 34 seconds

and have that the negative side running through the hole. That is a very fast way to make your boat disappear via electrolysis. Guaranteed problems.

33:44

33 minutes, 44 seconds

always run a ground wire back to the battery or to a bus bar that is only grounded to the battery and is isolated

33:52

33 minutes, 52 seconds

from the hole. The only place that the negative should be touching the electrical system is through the

33:59

33 minutes, 59 seconds

outboard because where the outboard mounts right there that is going to ground the hole to the battery but you don't have electrical current going

34:08

34 minutes, 8 seconds

through that because the outboard is also directly connected to the battery.

Chapter 10: Outro

34:12

34 minutes, 12 seconds

Well, that is going to wrap up this video. Hopefully, you learned something.

34:15

34 minutes, 15 seconds

Hopefully, I helped you make a decision on what kind of boat you want for your use case and certain things to look for,

34:23

34 minutes, 23 seconds

features to look for in a used boat or I guess a new boat. Just remember, a boat's only new once. Once you start

34:30

34 minutes, 30 seconds

using it, they're all all aluminum boats are going to have all of the same problems. Knowing about them and being able to mitigate them, being able to

34:38

34 minutes, 38 seconds

take care of your investment so you can make more memories on the water is extremely important. And of course, when you're buying a used boat, make sure that previous owner took care of it like he was going to have it forever as well.

34:48

34 minutes, 48 seconds

Thanks for watching. Don't forget to like, comment, and subscribe. And I will see you on the next video. Bye.